NITRO-SDK

Wireless Communication Tutorial

Version 1.0.0

Table of Contents

1	Overview of the Wireless Manager	5

1.1	Role of the Wireless Manager	5

1.2	MP Communication Protocol	5

1.3	Data Sharing	6

2	Operating the Wireless Manager	7

2.1	Organization of the Wireless Manager	7

2.2	Transitioning Between Internal States	8

3	Implementing the dataShare-Model	9

3.1	Initializing	10

3.2	Selecting the Communication Channel	11

3.3	Connecting in Parent and Child Modes	12

3.4	Processing Synchronously	12

3.5	Disconnecting and Terminating Processing	14

4	WH Library	15

4.1	Function Reference (Initialization, Termination, Reset)	15

4.1.1	WH_Initialize Function	15

4.1.2	WH_Finalize Function	15

4.1.3	WH_Reset Function	16

4.2	Function Reference (Connection)	16

4.2.1	WH_ForceChannel Function	16

4.2.2	WH_Connect Function	16

4.3	Function Reference (MP Communication)	17

4.3.1	WH_SetReceiver Function	17

4.3.2	WH_SendData Function	17

4.4	Function Reference (Data Sharing)	18

4.4.1	WH_StepDS Function	18

4.4.2	WH_GetSharedDataAdr Function	18

4.5	Function Reference: Key Sharing	18

4.5.1	WH_GetKeySet Function	18

4.6	Function Reference: Get State	19

4.6.1	WH_GetAllowedChannel Function	19

4.6.2	WH_GetConnectMode Function	19

4.6.3	WH_GetBitmap Function	19

4.6.4	WH_GetSystemState Function	19

4.6.5	WH_GetLastError Function	20

5	Appendix	21

5.1	WH_StateInXXXX and WH_StateOutXXXX Functions	21

5.1.1	Parent/Child Shared Functions in WH and WM	21

5.1.2	Parent Functions in WH and WM	22

5.1.3	Child Functions in WH and WM	23

Revision History

| Version | Revision Date | Description |
| --- | --- | --- |
| 1.0.0 | 11/24/2004 | Initial version. |

# Overview of the Wireless Manager

## Role of the Wireless Manager

The wireless manager (WM) is situated between the wireless communication hardware and applications. It receives information and directly shares that information with the hardware. The wireless manager is a library providing relatively low-level parts.

The wireless manager library is mainly an implementation of a game-specific wireless communication method. The wireless manager library provides a unique protocol called the MP communication protocol. Frameworks are also included, such as a data sharing function that operates on that protocol.

This document explains the fundamentals needed to use the wireless manager. This document also includes explanations, taking sample programs as examples and implementing them in real applications.

## MP Communication Protocol

As with the programming manual, there are three separate ways to use the DS wireless function depending on your purposes. The three modes are:

- Infrastructure

- DS Wireless Play

- Single-Card Play

This tutorial addresses only the DS Wireless Play mode.

In DS Wireless Play mode, communication occurs wirelessly while each connected device has a game card inserted. In Single-Card Play mode, a game card is inserted in one machine, and the other machines operate by downloading the program from that machine. In Infrastructure mode, communication occurs using the Internet.

When communicating wirelessly after a program downloaded in Single-Card Play mode has started, communication occurs in either the DS Wireless Play or Infrastructure mode.

The protocol normally used in DS Wireless Play mode is called the MP (Multi Poll) Communication protocol. This protocol provides the functionality called sending and receiving data in real-time with multiple machines, which is necessary in many communication game applications.

Using the MP communication protocol, communication occurs in the following steps as one cycle:

- The parent delivers (broadcasts) data to all children.

- All children return a response to the parent.

- The parent notifies (broadcasts) that the communication cycle is finished.

Note that each child communicates directly with only the parent and does not communicate directly with other children. Also, the real-time aspect is a priority. As such, another feature is that instead of being able to perform communication of one to several cycles in one picture frame (1/60th of a second), the amount of sendable/receivable data in one cycle is comparatively small.

## Data Sharing

Data sharing is a communication method for realizing on the MP communication protocol a technique called sharing data in real-time with all communicating devices, used frequently by game applications. This technique is realized in such a way that the parent collects data from each child, lumps it together, and then delivers it to all children as shared data.

Pulling this together (as in the steps mentioned in section 1.2), the steps are:

- The parent distributes shared data to all children.

- Each child responds with its own specific information to the parent.

- The parent collects returned information as shared data for the next send.

Note that the shared data received by each child is the data the parent collected from each child in the previous cycle.

Key sharing treats each device’s key data as shared data.

Data sharing is one sample application of the MP communication protocol. Key sharing is one example of how to use data sharing. These three terms should not be spoken of on the same level but, depending on the circumstances, they may be described in parallel in the manual or sample programs. Do not confuse these terms. This manual primarily covers data sharing.

# Operating the Wireless Manager

## Organization of the Wireless Manager

In the Nintendo DS, the wireless communication unit is connected to the ARM7 bus (see the hardware block diagram in the "NITRO Programming Manual"). In other words, the wireless communication unit is under the control of the subprocessor (ARM7).

Therefore, to control the communication function from the main processor (ARM9) in a normal game, an application calls WM APIs, which in turn issure commands to the subprocessor (ARM7). Many WM-related APIs have been implemented as asynchronous functions for streaming requests to the ARM7 in FIFO. Because the result of the request is sent via FIFO, it is received by the main processor, causing the callback stored by the user to be invoked. This allows you to obtain the result.

In the sample programs covered in this tutorial, this issue request and receive results with callback operation is treated as one set. Serial processing is basically realized as follows (A and A’, and B and B’ are sets).

- Call function A to send a request to ARM7. (Make the callback set at this time A’.)

- Send notification that A’ was called and processing is complete. A’ calls function B to send the next request.

- B’ is called, which was set when calling B, and B’ calls C.

- (The same pattern repeats.)

## Transitioning Between Internal States

The ARM7 that controls wireless communication is a machine that goes back and forth between several internal states. The following figure shows the main internal states.

Figure 2-1 Transitions Between Wireless Communication Internal States

Only the states needed for this explanation appear in this diagram. There is a more detailed figure showing transitions between internal states in the "NITRO SDK Function Reference Manual."

Each of the transitions shown above with arrows has a corresponding function. You can advance the processing by calling them in order. With few exceptions, you can transition from a state only to a neighboring state connected by arrows. For example, you cannot transition in a single bound from IDLE to MP_CHILD.

# Implementing the dataShare-Model

This completes the discussion of the fundamentals necessary for understanding the SDK sample program (dataShare-Model). Additional considerations of the sample program follow.

The sample program implements data sharing using the WH library wrapper libraries, wh.h and wh.c (hereafter referred to as the WH library). The functions needed to implement normal wireless communication programs are gathered in the WH library.

This section explains how to use the WH library for sample program tasks, such as:

- Initializing

- Selecting the Communication Channel

- Connecting in Parent and Child Modes

- Processing Synchronously

- Disconnecting and Terminating Processing

## Initializing

This section describes the procedure to implement data sharing using the WH library.

First, define the type of data to share. When the maximum number of connected children is 15, a maximum of 12 bytes can be shared using data sharing. (The sharable data size varies with the maximum number of connected children.) The data size must not exceed 12 bytes.

typedef struct ShareData_ {

u8  macadr[4];  // MAC address

u32 count;      // frame count

u16 level;      // radio reception strength

s16 data;       // graph display information

}ShareData;

Next, confirm the shared data region for send/receive on the program side. For the receive buffer, the region (in bytes) must be at least (shared data size x (maximum number of connected children + 1) ).

static u8         sSendBuf[256] ATTRIBUTE_ALIGN(32);

static u8         sRecvBuf[256] ATTRIBUTE_ALIGN(32);

Next, set the V-blank interrupt. The V-blank interrupt is needed in section "3.4 Processing Synchronously."

// interrupt setting

OS_SetIrqFunction( OS_IE_V_BLANK , VBlankIntr );

(void)OS_EnableIrqMask( OS_IE_V_BLANK );

(void)GX_VBlankIntr( TRUE );

(void)OS_EnableIrq();

(void)OS_EnableInterrupts();

In the WH_Initialize function, the OS_Alloc function is used to allocate the buffer.

- Call the OS_InitAlloc function and initialize the memory allocation system.

- Create a heap in main memory using the OS_CreateHeap function.

- Set the created heap to the current heap using the OS_SetCurrentHeap function.

Static void InitializeAllocateSystem(void)

{

void*        tempLo;

OSHeapHandle hh;

tempLo = OS_InitAlloc(OS_ARENA_MAIN,

OS_GetMainArenaLo(),

OS_GetMainArenaHi(),

1);

OS_SetArenaLo(OS_ARENA_MAIN, tempLo);

hh = OS_CreateHeap(OS_ARENA_MAIN ,

OS_GetMainArenaLo(),

OS_GetMainArenaHi());

if(hh < 0){

OS_Panic("ARM9: Fail to create heap...\n");

}

hh = OS_SetCurrentHeap(OS_ARENA_MAIN, hh);

}

Complete the necessary preparations on the program side and use WH_Initialize to initialize wireless communications.

The WH_Initialize function allocates the send/receive data buffer necessary for wireless communication and performs all processing necessary to initialize the wireless hardware. We recommend using the WH_Initialize function unless you want to perform detailed settings on the program side.

## Selecting the Communication Channel

In the sample program, obtain the wireless communication state using the WH_GetSystemState function in the main loop entered after the WH_Initialize function completes. The program branches to the respective process according to that state.

This the relevant portion of the sample program:

switch(WH_GetSystemState()) {

case WH_SYSSTATE_IDLE:

if(sChannelSelectMode){

ModeSelectChannel();

}else{

ModeSelect();

}

break;

case WH_SYSSTATE_ERROR:

ModeError();

break;

case WH_SYSSTATE_BUSY:

case WH_SYSSTATE_SCANNING:

ModeBusy();

break;

case WH_SYSSTATE_CONNECTED:

case WH_SYSSTATE_DATASHARING:

sShareDataPtr->count = sFrame;

if((WH_GetConnectMode() == WH_CONNECTMODE_MP_PARENT)

|| (WH_GetConnectMode() == WH_CONNECTMODE_DS_PARENT)){

ModeParent();

}else{

ModeChild();

}

break;

}

If initialization succeeds, the WH_SYSSTATE_IDLE state is entered when the WH_Initialize function completes.

The first thing called is the ModeSelect routine. In the ModeSelect routine, you select from the following three modes: PARENT, CHILD, and Select Channel Manually. When you select the manual mode, the ModeSelectChannel routine is called by the main loop, and the manually selected channel is forcibly set to the wireless communication channel.

Obtain the list of available communication channels using the WH_GetAllowedChannel function. Then call the WH_ForceChannel function with the selected channel as the argument to forcibly use the selected channel. When called with channel 0 as the argument, the empty communication channel is selected automatically during the parent connection.

By selecting a communication channel, the main loop returns to the state in which the ModeSelect routine is called.

## Connecting in Parent and Child Modes

When selecting PARENT with the ModeSelect routine, the connection is made in parent mode. When selecting CHILD, the connection is made in child mode.

To connect in the parent mode of data sharing, call the WH_Connect function with WH_CONNECTMODE_DS_PARENT as the argument.

To connect in the child mode of data sharing, call the WH_Connect function with WH_CONNECTMODE_DS_CHILD as the argument.

if(sKey.trg & PAD_BUTTON_A){

WH_Connect(WH_CONNECTMODE_DS_PARENT);

}else if(sKey.trg & PAD_BUTTON_B){

WH_Connect(WH_CONNECTMODE_DS_CHILD);

}else if(sKey.trg & PAD_BUTTON_X){

sChannelSelectMode = 1;

}

The WH_Connect function can be called in either mode, but the procedure used for connecting is different. In the WH library, the difference is absorbed in the library, so on the program side you will not notice the difference in the connection procedure, except that a different argument is passed.

## Processing Synchronously

When the connection completes normally in the parent or child mode of data sharing, the state obtained with the WH_GetSysState function transitions to WH_SYSSTATE_DATASHARING (data sharing).

For stable wireless communication, you must call the synchronous processing function WH_StepDS before starting the MP communication cycle of that frame. For the V-count where the WM library prepares MP communication, the parent is 260 and the child is 240 by default. It is designed to be as efficient as possible when called with the V-blank interrupt (V-count is 192). With this in mind, WH_StepDS is called in the V-blank interrupt handler function in the sample program.

static void VBlankIntr( void )

{

if(WH_GetSystemState() == WH_SYSSTATE_DATASHARING)

{

if(WH_StepDS(sSendBuf)){

u16 i;

u8 *adr;

for(i = 0; i < WM_NUM_MAX_CHILD + 1; ++i){

ShareData *sd;

adr = (u8 *)WH_GetSharedDataAdr(i);

sd = (ShareData *)&(sRecvBuf[i * sizeof(ShareData)]);

if(adr != NULL){

MI_CpuCopy8(adr, sd, sizeof(ShareData));

sRecvFlag[i] = TRUE;

}else{

sd->level = 0;

sd->data = 0;

sRecvFlag[i] = FALSE;

}

}

}else{

u16 i;

for(i = 0; i < WM_NUM_MAX_CHILD + 1; ++i){

sRecvFlag[i] = FALSE;

}

}

}else{

u16 i;

for(i = 0; i < WM_NUM_MAX_CHILD + 1; ++i){

sRecvFlag[i] = FALSE;

}

}

OS_SetIrqCheckFlag(OS_IE_V_BLANK);

}

For data shared by synchronous processing, obtain the top address using the WH_GetSharedDataAdr function and copy it to the receive data region allocated on the program side.

In the main loop, use the WH_GetConnectMode function to determine the connection mode. If in the parent mode, call the ModeParent routine. If in the child mode, call the ModeChild routine. The send/receive results are displayed in each routine.

The link strength icon is also displayed, which graphically shows the strength of the communication link while sharing data.

## Disconnecting and Terminating Processing

When you want to disconnect a specific child from its parent with a user operation, use the WM_Disconnect function. When you want to disconnect multiple or all children at once, use the WM_DisconnectChildren function.

Using the WH library, when you want to terminate wireless communication by calling the WH_Finalize function, you can perform the appropriate end processing by evaluating the connection mode and the current WH library state. With the WH_Finalize function, wireless communication transitions to the IDLE state. From there, by sequentially calling the WM_PowerOff, WM_Disable, and WM_Finish functions (or the WM_End function to perform all three), you can completely finish, including disconnecting the wireless communication hardware.

# WH Library

The WH library is a collection of functions needed to implement normal wireless communication programs. These functions are contained in the wrapper libraries, wh.h and wh.c. This section explains the functions collected in the WH library (including functions not used in the sample programs).

## Function Reference (Initialization, Termination, Reset)

This section discusses the functions that initialize, terminate, and reset

### WH_Initialize Function

C Specification:	int WH_Initialize(void);

Arguments:		None

Return Values:		TRUE	Success

FALSE	Failure

Normally, WH_Initialize automatically allocates the communication send/receive data buffer necessary for wireless communication and initializes wireless communication hardware. The wireless communication state transitions, once the wireless communication state becomes IDLE. When TRUE is returned, the WM_Init function succeeded, and the WM_Enable function was called successfully. Initialization processing is complete when the return value of the WH_GetSystemState function becomes WH_SYSSTATE_IDLE.

You must create a heap in the main memory for the internally-called OS_Alloc function.

### WH_Finalize Function

C Specification:	int WH_Finalize(void);

Arguments:		None

Return Values:		TRUE	Success

FALSE	Failure

WH_Finalize calls the appropriate end process determined from the WH library state and the connection mode. The wireless communication state transitions to IDLE after processing completes. When TRUE is returned, the function call for end processing succeeded. The processing ends when the return value of the WH_GetSystemState function becomes WH_SYSSTATE_IDLE.

To completely terminate wireless communication, you must call the WM_PowerOff, WM_Disable, and WM_Finish functions, or just the WM_End function.

### WH_Reset Function

C Specification:	int WH_Reset(void);

Arguments:		None

Return Values:		TRUE	Success

FALSE	Failure

WH_Reset transitions the wireless communication state to IDLE regardless of the current state (such as connection mode). When TRUE is returned, the WH_Reset call succeeded. The process completes when the return value of WH_GetSystemState becomes WH_SYSSTATE_IDLE.

## Function Reference (Connection)

This section discusses the two connection functions, WH_ForceChannel and WH_Connect.

### WH_ForceChannel Function

C Specification:	void WH_ForceChannel(u16 ch);

Arguments:		ch	Communication channel

Return Values:		None

WH_ForceChannel specifies the communication channel to use when starting communication as a parent.

When ch is set to 0, it automatically searches for and uses an unused communication channel when connecting.

### WH_Connect Function

C Specification:	int WH_Connect(int connectmode);

Arguments:		connectmode	Connection mode

Return Values: 	TRUE		Success

FALSE		Failure

Connection code definitions:

enum {

WH_CONNECTMODE_MP_PARENT,  // Parent MP connection mode

WH_CONNECTMODE_MP_CHILD,   // Child MP connection mode

WH_CONNECTMODE_KS_PARENT,  // Parent key-sharing connection mode

WH_CONNECTMODE_KS_CHILD,   // Child key-sharing connection mode

WH_CONNECTMODE_DS_PARENT,  // Parent data-sharing connection mode

WH_CONNECTMODE_DS_CHILD,   // Child data-sharing connection mode

WH_CONNECTMODE_NUM

};

WH_Connect specifies the connection mode and starts the wireless communication connection. It automatically finds parent and child processing differences and transitions to data sharing and key sharing. When TRUE is returned, the function call for connection processing succeeded. Whether for a parent or child, the process completes when the WH_GetSystemState function returns the following return values: MP connection is WH_SYSSTATE_CONNECTED, data sharing is WH_SYSSTATE_DATASHARING, and key sharing is WH_SYSSTATE_KEYSHARING.

## Function Reference (MP Communication)

This section discusses the two MP communication functions, WH_SetReceiver and WH_SendData.

### WH_SetReceiver Function

C Specification:	void WH_SetReceiver(WHReceiver proc);

Arguments:		proc	WHReceiver type callback function

Return Values: 	None

WHReceiver type definitions:

typedef void (*WHReceiver)(u16 aid, u16* data, u16 size);

WH_SetReceiver sets the MP communication data receive callback function. There is no need to set this when data sharing or key sharing.

The send source aid, received data, and receive data size are passed to the callback function.

### WH_SendData Function

C Specification:	int WH_SendData(

void *data, u16 datasize, WHSendCallbackFunc callback);

Arguments:	data		top address of send data

datasize	size of send data

callback	WHSendCallbackFunc type callback function

Return Values:	TRUE		Success

FALSE		Failure

WHSendCallbackFunc type definitions:

typedef void (*WHSendCallbackFunc)(BOOL result);

WH_SendData starts an MP communication data send. You do not need to call this when data sharing or key sharing. When TRUE is returned, the WM_SetMPDataToPortEx function call succeeded. The process completes when the callback function is called.

The send results are passed to the callback function. You must not change the contents of the send data buffer until the callback function is called.

## Function Reference (Data Sharing)

This section discusses the two data sharing functions, WH_StepDS and WH_GetSharedDataAdr.

### WH_StepDS Function

C Specification:	int WH_StepDS(void *data);

Arguments:		data	top address of send data

Return Values:		TRUE	Success

FALSE	Failure

WH_StepDS advances data sharing synchronicity by one. When TRUE is returned, the process is complete. To get shared data, use the WH_GetSharedDataAdr function.

For stable wireless communication, you must call this function before starting the MP communication cycle of that frame. We recommend calling it shortly after starting the V-blank interrupt.

### WH_GetSharedDataAdr Function

C Specification:	u16 *WH_GetSharedDataAdr(u16 aid);

Arguments		aid	aid of child you want to get shared data for

Return Values: 	top      address of shared data of specified child

NULL     is returned when it fails.

Call WH_GetSharedDataAdr when you want to get data sharing shared data with the child aid.

## Function Reference: Key Sharing

This section discusses the key sharing function, WH_GetKeySet.

### WH_GetKeySet Function

C Specification: 	int WH_GetKeySet(WMKeySet *keyset);

Arguments:		keyset	pointer to buffer that stores shared key data

Return Values:		TRUE	Success

FALSE	Failure

WH_GetKeySet stores key data shared with key sharing in the buffer. The process is complete when TRUE is returned.

For stable wireless communication, the WH_GetKeySet function must be called before starting MP communication cycle of that frame. We recommend calling after starting the V-blank interrupt.

## Function Reference: Get State

This section discusses the get state functions: WH_GetAllowedChannel, 
WH_GetConnectMode, WH_GetBitmap, WH_GetSystemState, and WH_GetLastError.

### WH_GetAllowedChannel Function

C Specification:u16 WH_GetAllowedChannel(void);

Arguments:		None

Return Values: 	bit pattern of communication channels permitted for use

Internally, WH_GetAllowedChannel calls the WM_GetAllowedChannel function.

### WH_GetConnectMode Function

C Specification:			int WH_GetConnectMode(void);

Arguments:		None

Return Values: 	the set connection mode

WH_GetConnectMode returns the connection mode set as an argument with the WH_Connect function. The return values are undetermined until WH_Connect is called. Until the next time WH_Connect is called, the previously-set connection mode is returned.

### WH_GetBitmap Function

C Specification:		u16 WH_GetBitmap(void);

Arguments:		None

Return Values: 	bit pattern showing connected terminal

The bit corresponding to the connected terminal is set to 1. The lowest bit corresponds to the parent (aid=0), and the highest bit corresponds to the 15th child (aid=15).

### WH_GetSystemState Function

C Specification:	int WH_GetSystemState(void);

Arguments:		None

Return Values:		Internal state of WH library

Definitions of WH library internal states:

enum {

WH_SYSSTATE_STOP,          // initial state

WH_SYSSTATE_IDLE,          // standing by

WH_SYSSTATE_SCANNING,     // scanning

WH_SYSSTATE_BUSY,          // connecting

WH_SYSSTATE_CONNECTED,    // connection complete (communication is possible in this state)

WH_SYSSTATE_DATASHARING, // connected with data-sharing enabled

WH_SYSSTATE_KEYSHARING,  // connected with key-sharing enabled

WH_SYSSTATE_ERROR,        // error has occurred

WH_SYSSTATE_NUM

};

WH_GetSystemState obtains the current internal state of the WH library.

### WH_GetLastError Function

C Specification:		int WH_GetLastError(void);

Arguments:			None

Return Values:			error code

Definitions of error codes:

enum {

// your own error codes

WH_ERRCODE_DISCONNECTED = WM_ERRCODE_MAX, // disconnected from parent

WH_ERRCODE_PARENT_NOT_FOUND,                 // no parent

WH_ERRCODE_NO_RADIO,                           // wireless use not possible

WH_ERRCODE_LOST_PARENT,                       // parent not found

WH_ERRCODE_MAX

};

WH_GetLastError obtains the details of the error that just occurred.

# Appendix

## WH_StateInXXXX and WH_StateOutXXXX Functions

As stated in "2.1 Organization of the Wireless Manager," the Wireless Manager API performs wireless communication with a combination of the request send function (call function) and the callback function that receives notification.

For internal functions in the WH library, the request send function is WH_StateInXXXX and the callback function is WH_StateOutXXXX.

### Parent/Child Shared Functions in WH and WM

Table 5-1 Corresponding WM Functions (Parent/Child Shared Functions)

| WH Library Function Names | Corresponding Wireless Manager Function |
| --- | --- |
| WH_StateInInitialize() | WM_Init() |
| WH_StateInEnable() WH_StateOutEnable() | WM_Enable() |
| WH_StateInPowerOn() WH_StateOutPowerOn() | WM_PowerOn() |
| WH_StateInReset() WH_StateOutReset() | WM_Reset() |
| WH_StateInSetMPData() WH_StateOutSetMPData() | WM_SetMPDataToPortEx() |
| WH_StateInPowerOff() WH_StateOutPowerOff() | WM_PowerOff() |
| WH_StateInDisable() WH_StateOutDisable() | WM_Disable() |

### Parent Functions in WH and WM

Table 5-2  Corresponding WM Functions (Parent Functions)

| WH Library Function Names | Corresponding Wireless Manager Function |
| --- | --- |
| WH_StateInMeasureChannel() WH_NextMeasureChannel() WH_StateOutMeasureChannel() | WM_GetAllowedChannel() WM_MeasureChannel() |
| WH_StateInSetParentParam() WH_StateOutSetParentParam() | WM_SetParentParameter() |
| WH_StateInStartParent() WH_StateOutStartParent() | WM_StartParent() |
| WH_StateInStartParentMP() WH_StateOutStartParentMP() | WM_StartMP() WM_StartDataSharing() is also called when in data sharing mode. |
| WH_StateInStartParentKeyShare() WH_StateOutStartParentKeyShare() | WM_StartKeySharing() |
| WH_StateInEndParentKeyShare() WH_StateOutEndParentKeyShare() | WM_EndKeySharing() |
| WH_StateInEndParentMP() WH_StateOutEndParentMP() | WM_EndMP() |
| WH_StateInEndParent () WH_StateOutEndParent () | WM_EndParent() |
| WH_StateInDisconnectChildren() WH_StateOutDisconnectChildren() | WM_DisconnectChildren() |

### Child Functions in WH and WM

Table 5-3  Corresponding WM Functions (Child Functions)

| WH Library Function Names | Corresponding Wireless Manager Function |
| --- | --- |
| WH_StateInStartScan() WH_NextScan () WH_StateOutStartScan() | WM_GetAllowedChannel() WM_StartScan() |
| WH_StateInEndScan() WH_StateOutEndScan() | WM_EndScan() |
| WH_StateInStartChild() WH_StateOutStartChild() | WM_StartConnect() |
| WH_StateInStartChildMP() WH_StateOutStartChildMP() | WM_StartMP() WM_StartDataSharing() is also called when in data sharing mode. |
| WH_StateInStartChildKeyShare() WH_StateOutStartChildKeyShare() | WM_StartKeySharing() |
| WH_StateInEndChildKeyShare() WH_StateOutEndChildKeyShare() | WM_EndKeySharing() |
| WH_StateInEndChildMP() WH_StateOutEndChildMP() | WM_EndMP() |
| WH_StateInEndChild () WH_StateOutEndChild () | WM_Disconnect() |

© 2004 Nintendo

The contents of this document cannot be duplicated, copied, reprinted, transferred, distributed or loaned in whole or in part without the prior approval of Nintendo.