NITRO-SDK

Using the Wireless Communication API
(DRAFT)

Version 0.3.2

The contents in this document are highly confidential and should be handled accordingly.

Table of Contents

1	Wireless Communication API Overview	4

1.1	Introduction	4

1.2	Basic Specifications of the Wireless Hardware	4

1.3	Configuration of the Wireless Library	4

2	Glossary	5

3	Local Game Mode	9

3.1	Wireless Protocol Specifications (802.11 Specification and MP Protocol)	9

3.1.1	Overview	9

3.1.2	Connecting	10

3.1.3	Communications	11

3.2	Wireless Protocol Specifications (Library Level)	13

3.2.1	Communication Packet	13

3.3	Cautions when Using the Wireless API	15

3.3.1	The burden of wireless usage	15

3.3.2	The Internal State of the Wireless Library	15

3.3.3	Asynchronous Functions and Callbacks	15

3.3.4	Usage Channels	16

3.3.5	Connection Information	16

3.3.6	Port Communications	17

3.3.7	Priority and Send Queue	18

3.3.8	The Maximum Number of Send Bytes	18

3.3.9	Key Sharing and Data Sharing	19

3.3.10	Events Triggered by the Wireless Library	21

3.4	FAQ	25

3.4.1	Initialization process	25

3.4.2	Connection process	25

3.4.3	General MP communications	27

3.4.4	Data Sharing	27

3.4.5	Others	28

3.5	Important Notes for Recent Releases	29

3.5.1	Changes to Receive Processing (SDK 2.0 PR4 and Higher)	29

3.5.2	Changes to the Key Sharing Packaging (SDK 2.0 PR4 and Higher)	30

3.5.3	Changes to the Notification Error Code for Transmission Failure (SDK 2.0 PR6 and Higher)	30

3.5.4	New Notification: WM_ERRCODE_INVALID_POLLBITMAP (SDK 2.0 PR6 and Higher)	30

3.5.5	Notification Changes for Port Receiving Callback Connections and Disconnections (2.0 PR7 & Higher)	30

3.5.6	Changes to the WMParentParam and WMGameInfo Structures (from 2.0 RC1)	30

3.5.7	Change in MAC address type (from NitroSDK Version 2.0 RC1)	31

3.5.8	Corrected the wireless demo (NITRO-SDK 2.0 RC2 and as needed)	31

# Wireless Communication API Overview

## Introduction

The Nitro SDK includes a set of APIs for wireless communications. This document describes the basic features of the wireless API.

This document is a draft. It was released to provide information in a timely fashion, and we cannot guarantee its accuracy or completeness. If you find any errors, please notify us via support@noa.com .

## Basic Specifications of the Wireless Hardware

The basic specifications of the wireless hardware in the Nintendo DS are shown below.

- Equivalent to IEEE 802.11:

  - Frequency band: 2.4 GHz

  - Communication speed: 2 Mbps (theoretical value)

- Communication range: About 20 meters (depending on the communication environment)

## Configuration of the Wireless Library

The NitroSDK wireless library is divided into two sections: the ARM9 library and ARM7 library. The ARM7 library controls the wireless hardware and the ARM9 library transmits requests from the application to the ARM7. When building an application, you can ignore most specifications on the ARM7 side but, in some cases, you should be aware of cache-related processing issues.

The wireless library provides two main communication modes.

| Mode | Description |
| --- | --- |
| Local game mode | (The DS wireless communication mode) Allows wireless networked play with up to 16 nearby DS devices. |
| Infrastructure mode | Allows connection to the Internet via general wireless access points supporting the 802.11 standard. |

This document does not describe support for infrastructure mode. For details, please contact us.

# Glossary

This glossary defines the terms used in this guide.

| Term | Definition |
| --- | --- |
| 1 session | The period of time from a single WM_StartParent to WM_EndParent. |
| 802.11 | A wireless communication standard defined by the IEEE. It defines a wireless communication method that permits speeds of up to 2 Mbps. (802.11b, which is in the same family of standards, allows a maximum speed of 11 Mbps, and is a popular standard for wireless PC communications. 802.11b is backwards compatible with 802.11.) The DS uses the 1–2 Mbps backward compatibility mode defined in the 802.11b standard. |
| AID | Association ID. The parent's AID is always 0. Child devices are assigned AIDs from 1 to 15 when they connect. If the maximum number of children permitted to connect is set to n, assigned AIDs must be from 1 to n. |
| Beacon | A wireless signal, in addition to an MP sequence, sent periodically by the parent. A child can receive the beacon even when it has not established a connection. A child making a new connection selects the parent based on the GameInfo contained in the beacon. Normally sent at intervals of several hundred ms. |
| Block transmission | Feature designed to transmit a chunk of data from the parent to multiple children at the same time. For details, see WBT* in the "NITRO-SDK Function Reference Manual." |
| BSS | Basic Service Set. Specifies a set that performs transactions for a single service. In local game mode, refers to the group that includes a parent and the children connected to it. |
| BSSID | Basic Service Set ID. In local game mode, the BSSID identifies each parent device when connecting. The parent's MAC address is used as the BSSID. |
| Channel | A portion of the communication band. Based on the 802.11 standard, 1 to 14 channels can be used, but the actual number of available channels is limited by regulations in each country. Also, adjacent channels can interfere with each other, so we recommend setting up channels with roughly five intervals between them. |
| Child device | The devices (1–15) that connect to a parent device in local game mode. |
| Continuous communication mode | The communication mode sustained using continuous MP sequences. (This contrasts with frame-synchronous communication mode.) It is effective for sending large amounts of data, but consumes lots of power and should not be used for long periods. It differs from frame-synchronous communications mode in that it starts MP sequences continuously, but from a control standpoint, the two modes are virtually identical. Thus, a single program can switch between the two modes. |
| Data sharing | Enables the sharing of data with a user-defined size in addition to key input. |
| Single-Card play | A child device without a Game Pak or Game Card can boot by downloading a game's ROM image from the parent over a wireless connection. For details, see MB_* in the reference manual. |
| Frame-synchronous communication mode | The communication mode that synchronizes MP communications and the picture frame. (This contrasts with continuous communications mode.) The number of times to communicate during each picture frame is specified. However, if signal conditions are poor and a resend is necessary, the transmission is resent without synchronizing with the frame (if possible). |
| Game frame | The period defining a unit of game processing. If a game is running at 30 frames/second, the game frame is 1/30 of second. |
| GameInfo | A data structure that indicates the type of game offered by a parent and contains the data needed to connect. Contains the GGID, the TGID, the maximum number of bytes the parent can send, etc. It can also contain user-defined information. For example, with Single-Card play, the GameInfo contains the game name, icon data, etc. |
| GGID | Game Group ID. A unique four-byte ID assigned by Nintendo to each game title and series title. Used when connecting. |
| Indication | A notification sent automatically from the wireless hardware to the application in response to receiving data or another event. Contrast an indication with a response to a request from the application. |
| Key response frame | The type of frame a child uses to respond to an MP frame from the parent. |
| Key sharing | Functionality common to key input data. Enables you to use wireless communications without worrying about the details. |
| MAC address | An ID number for the wireless hardware. Each DS device has a different 6 byte MAC address. |
| MP communications | A generic term for communications using multi-poll (MP) sequences. In some cases, it refers to the communication for a single MP sequence. |
| MP frame | The frame at the beginning of an MP sequence, in which the parent broadcasts to the child. |
| MP sequence | Multi-Poll sequence. Nintendo's proprietary extension of 802.11 enables wireless communications with a low latency time. For details, see the chapter on Local Game Mode. |
| MP_ACK frame | The frame broadcast by a parent to its children at the end of an MP sequence. |
| Null response frame | The frame a child uses to respond to an MP frame sent by the parent. Sent when the response data cannot be sent within the given time constraints. |
| Packet | A unit of communication that contains a header and footer. It contains a port number, a packet size and, when necessary, destination information, a sequential number, and so on. In actual communications, within a single MP, a payload contains multiple packets. |
| Parent device | The device that controls all communication in local game mode. |
| Payload | The area in the MP and key response frames that carries data. |
| Picture frame | The time that elapses from one V-Blank synchronization signal to another (1/60 of a second). |
| PollBitmap | A 16-bit bitmap in which each bit corresponds to the AID of a child device. In an MP frame, the bits of the children from which a response is expected are enabled. In an MP_ACK frame, the bits of the children from which the parent received no response are enabled. |
| Port | A concept used in the wireless library to realize multiple communication channels. If a transmission specifies a port that is an integer from 0 through 15, the destination calls a callback that corresponds to the number. Note that this port has a lower level of abstraction than ports used in TCP/IP. |
| Raw communication | A communication method that does not perform additional controls like those in sequential communications. Data may not reach its destination and the same data may be transmitted several times. If ports 0 through 7 are selected, raw communication is used. |
| Sequential communication | The upper layer of MP communications, which guarantees the integrity of communication. The wireless library uses sequential numbers to eliminate long packets and insure that packets reach their destination. If ports 8 through 15 are selected, sequential communication is used. |
| SSID | Info used to screen children connecting to a parent. The child uses the GGID and TGID from GameInfo to generate an SSID. The parent connects only to children having a matching GGID and TGID based on the SSID. The lower half of the user area that is not used for matching can be used for communications from the child to the parent. |
| TGID | Temporary Group ID. A two-byte ID assigned when a new game or session is started. Used by a single device to distinguish between new and old sessions of a parent. |

# Local Game Mode

## Wireless Protocol Specifications (802.11 Specification and MP Protocol)

### Overview

In local game mode, the network is configured in a star shape. In this mode, communication is limited to that between parent and children and children cannot communicate with each other. However, a parent can transmit data to multiple children at the same time.

### Connecting

The process leading up to a connection is as follows:

|  | Parent | Child |
| --- | --- | --- |
| 1 | Initialize the wireless hardware. | Initialize the wireless hardware. |
| 2 | Set the parent's GGID, TGID, and other data for communications. | Scan the beacon on the specified channel and find the available parent devices. |
| 3 | Send the beacon on the specified channel. The beacon's GameInfo contains the GGID, the TGID, and a flag indicating that is it available for connection. |  |
|  |  | Based on the data in GameInfo, list the parent devices for the user and prompt for a selection. |
|  |  | Generate an SSID using the GGID and TGID contained in GameInfo, and connect to the parent based on the BSSID (the parent's MAC address) and the SSID. |
| 4 | Compare the SSID of the incoming child with its own GGID and TGID, and OK the connection if there is a match. |  |
| 5 | Assign an AID to the child and complete the connection. | Receive the AID from the parent and complete the connection. |

### Communications

The operations flow for multi-poll (MP) communications, which is used for local game mode, is shown here.

When MP communication starts, the wireless library automatically synchronizes with the V-Blank. Note that while the V-Blank timing is adjusting, the V-Blank period is longer than 16.7 ms. Each frame grows by a maximum of 0.5 ms.

The main feature of MP communications is that the protocol completely controls the timing of the parent's transmission.

- First, the parent broadcasts an MP frame to all child devices.

The MP frame contains data sent from the parent to the child, but it also sends control data. It contains a PollBitmap, which indicates which children should respond, and a TXOP, which determines how many bytes of transmission time are assigned to the children. 
Data in the MP frame determines the time distribution of the overall MP sequence.

- Children that receive the MP frame examine the PollBitmap and TXOP, and send a key response frame to the parent after waiting the specified amount of time.

The key response contains a confirmation that the parent data was received as well as other data that must be sent to the parent. If the child attempts more data than fits within the transmission time allotted by the parent, the hardware automatically rejects the data and sends a null response frame, which just acknowledges receipt.

Because a key response frame is sent automatically by the hardware in response to an MP frame, the data sent in a key response frame must be stored in advance. Accordingly, the outgoing data content cannot be changed during a sequence in response to the incoming MP frame data. If no response data is stored, a null response frame is sent.

- Finally, the parent broadcasts an MP_ACK frame to all children to acknowledge receipt. 
The MP_ACK frame also contains a PollBitmap field, but unlike the field used in an MP frame, this field indicates the children from which neither a key response nor a null response was received. Each child confirms whether the parent received its transmission by looking at the PollBitmap in the MP_ACK frame to see if the bit indicating itself is enabled.

If PollBitmap is not enabled here, the successful transmission is guaranteed. If the AID bit of the child is enabled or if the child does not receive the MP_ACK frame within a certain period of time, it is assumed to be a transmission failure.

Because the wireless hardware consumes lots of power when active, it enters low-power mode during MP communication. This occurs automatically and normally the application can ignore the power mode. However, note that in continuous communications mode, MP sequences continue steadily and the hardware seldom switches to low-power mode. Accordingly, the device consumes lots of power.

When the parent receives no acknowledgment from a child, the flow of operations looks like this.

If reception fails, the MP_ACK frame indicates from which children no response was received, and then an MP sequence for a resend is started. Note the resend MP sequence targets only children from whom a response was not received. However, depending on the communication mode and the type of packet that was not received, no resend occurs and the application is notified only that a transmission failure occurred.

MP frame's PollBitmap indicates which children should respond. By controlling the PollBitmap from the start, you can have only the included children respond, and this reduces the total communication time. However, note that children not specified by the PollBitmap cannot send key response frames and are not given an opportunity to send data to the parent. Accordingly, during MP communications, bits for all connected children in the PollBitmap are enabled so that communication bandwidth is allocated for child communications. However, this is not true during a resend. The wireless library offers a mode that permits precise control of the PollBitmap, but an in-depth understanding of the wireless protocol is required to use it.

## Wireless Protocol Specifications (Library Level)

### Communication Packet

With MP protocol, the communications between parent and child are defined only up to the receipt of a chunk of data. In the wireless library layer, the actual control of communications is performed by the packet, which contains a data structure for communications control.

A single packet consists of a two-byte header, the data to be sent, and a footer (a maximum of four bytes for a parent and two bytes for a child).

The bit assignments for headers and footers are shown here.

The header contains the data length (in two-byte units), port number, and control flags. The footer contains a bitmap of the destination children as well as sequential numbers. If the data length is 0, it is treated as 512 bytes.

Packets sent from parent to children normally contain a destination bitmap. However, if the header's DestBitmap Flag is set to 0, then the parent is broadcasting to all children and the footer does not contain a destination bitmap.

Also, the sequential number is used to control sequential communication. If the highest-order bit of the header's four-bit port number is enabled (i.e. the port number is 8 or higher), then a sequential number is added.

During communications, multiple packets are included as permitted by the byte limit. The maximum number of bytes permitted in a child transmission is set by the parent at connection time, and children use the value specified by the parent without modification.

Packet

Packet

Example of packing multiple packets

Max. transmission size

Packet

Max. transmission size if only a single packet of max. size is used

Not included in max. transmission size

Note that the space required internally for each packet's header and footer is added to the maximum transmission size. The maximum transmission size should be viewed as the maximum number of bytes available for user data.

Accordingly, when sending multiple packets, additional space is used by the headers and footers between the chunks of data. For each packet, up to six additional bytes are required for a parent transmission and up to four additional bytes for a child transmission.

## Cautions when Using the Wireless API

This section contains information you must pay close attention to when using the Wireless API.

### The burden of wireless usage

Because the wireless driver in the SDK is currently more than 100 kilobytes in size, the wireless driver code cannot fit into the ARM7 working memory and must be stored in main memory. Accordingly, during wireless communications, the ARM7 must access main memory frequently, and during the operations that require the ARM9 to continuously access main memory, such as rendering, a large overhead can be generated. (Under normal conditions, the ARM7 has priority over the ARM9 in accessing main memory.)

If you give main memory access priority to ARM9 (using HDMA, etc.), frequent access to the main memory by ARM9 may actually hinder wireless driver execution. In particular, when accessing main memory with generic DMA, it is likely that ARM7 program execution will be delayed for long periods of time. If you are operating the wireless driver while ARM9 has access priority to main memory, if at all possible do not use generic DMA.

In the future, we may allocate VRAM-C or VRAM-D to the ARM7 for use by the wireless driver, in order to reduce the time that the ARM7 accesses main memory. When planning a resource-intensive application that uses wireless functionality, consider allocating part of VRAM to the ARM7 as one possibility.

### The Internal State of the Wireless Library

As described in $NITROSDK_ROOT\man\en_US\wm\wm\wm_state_diagram.html, the Wireless Library has an internal state. Depending on this state, there are limitations on the APIs that can be called. Calling WM_Init() after the Nintendo DS is started up causes the Wireless Library to immediately transition to the READY state. Refer to the reference file to see in which state each API can be called.

### Asynchronous Functions and Callbacks

Because the Wireless API must send instructions to the driver on the ARM7 side, most of its functions are asynchronous. The asynchronous API takes the WMCallbackFunc type callback as an argument, and the callback is called after the asynchronous processes have ended. If there are no asynchronous processes occurring, the API return value is always WM_ERRCODE_OPERATING.

WMCallbackFunc-type functions take the sole argument WMCallback* arg, but in several APIs, they pass structures unique to those APIs (Example: WMPortRecvCallback), so use them as needed after casting them in that type. $NITROSDK_ROOT\man\en_US\wm\wm\WMCallbackFunc.html contains information about which API returns which type of argument.

Note: The callback is called in the PXI interrupt handler. APIs that are in interrupt prohibited state cannot be used. Also, do not perform processes that take a long time. This might delay other interrupts on ARM9, and ARM7’s wireless driver when it is waiting for the end of the ARM9’s callback. This might have negative effects on the wireless processes.

### Usage Channels

The 802.11 standard allows for the use of one to fourteen channels, but there are also limitations imposed on the channels by regulations in other countries. Even if channels can be used, because neighboring channels can cause mutual interference, the channels used must be as far apart as possible.

The usable channels for the Nintendo DS are stored internally, and among them an API has been prepared that sets up channel groups a sufficient distance apart (WM_GetAllowedChannel). The application must select a channel to use from among the channels obtained with the API. It is left up to each application to determine which channel to select from among the available channels, but try to select a channel with low radio utilization. The radio utilization of the designated channel can be obtained with WM_MeasureChannel.

### Connection Information

While the parent device is in either the PARENT or the MP_PARENT state, a signal known as a beacon is being transmitted at a regular interval (the WMParentParam.beaconPeriod[ms] interval). The child device that wants to connect to a parent device obtains this beacon with WM_StartScan, and can connect to the parent device by leaving the included WMBssDesc structure as it is and passing the WM_StartConnect argument.

There are a variety of fields in WMBssDesc, but the ones that are important when a child device connects to a parent device are:

- WMBssDesc.bssid

- WMBssDesc.gameInfo.ggid

- tgid

The BSSID is an ID that identifies a parent device. It uses the MAC address of the parent device as-is in the Nintendo DS local game mode. The remaining GGID and TGID identify the services available from the parent device. The GGID is an ID allocated to each game or to each series (if communications are possible between the same series). Then the child device looks up the scan results using WMBssDesc.gameInfo.ggid, and confirms whether there is a parent device to which it can connect (a code for confirmation must be described on the application side). The parent assigns a unique TGID to each session, ensuring that each child connects to the session that corresponds to the service requested by the child.

For internal operations, when a child device connects to a parent device, it uses a BSSID and an SSID. The SSID is a total of 32 bytes, but in local game mode, the first 4 bytes of that SSID are the GGID, and the next 2 bytes are allocated to the TGID. Including a further 2 bytes of a reserved region, the 8 bytes are used as a service identifier. The parent device compares the first 8 bytes of the SSID declared by the child device to which it has connected with its own GGID and TGID, then confirms whether the child device is an appropriate partner. Inappropriate child devices are refused during the steps leading up to a connection.

The 8 bytes in the first half of the SSID are set automatically from the contents of the WMBssDesc.gameInfo.ggid and the tgid field inside WM_StartConnect, so the application programmer can ignore them. However, inside the SSID, the 24 bytes of the latter half not used as a service identifier are available to the application to use freely. As the ssid argument of WM_StartConnect, it is possible to freely set the user data and send it to the parent device. This can be received as WMStartParentCallback.ssid when the child device takes in the data set in the 24 bytes of the latter half of the SSID and the parent device gets the WM_STATECODE_CONNECTED notification in the WM_StartParent callback.

### Port Communications

To allow multiple layers of communication paths for MP communication, the wireless library adopted the concept of a port. The parent and child each have 16 virtual ports, and by specifying the port number to send data, the processes on the receiving end can be sorted out.

The receiving end sets the receiving callback function for the port with the specified number by using WM_SetPortCallback after the wireless initialization. Then, when MP communication receives the transmission data set with WM_SetMPDataToPort* by the sending end, the receiving callback that corresponds to the port number is called on the receiving end. When a new connection is made or the communication target disconnects itself, the receiving callbacks of all ports would be notified.

There are two types of port communications depending on the port number to use. Ports 0 to 7 are used for Raw communications, and ports 8 to 15 are used for Sequential communications. Raw communications do not use most of the communication controls. There are some cases where the data is not received by the target, or the multiple sets of the same data are received by the target. On the other hand, Sequential communication checks for duplication at the wireless library level by attaching sequence numbers to each packet and performing communication. By combining with the low-level retransmission processes, it performs communication securely and without duplication. Unlike the relationship between TCP and UDP, there is not much difference between Sequential communication and Raw communication in terms of latency or throughput. Make a selection according to the need for the retransmission. If there is a child with weak radio wave signals, note that the retransmission to that child may become a bottleneck for Sequential communication.

If the communication fails, with Raw communication, retransmission is attempted for the number of times specified by the argument defaultRetryCount of the WM_StartMPEx function. If MP communication had been started with WM_StartMP, it is assumed to be specified as 0 (no retransmission). With Sequential communication, retransmissions are repeated infinitely many times until it succeeds. When the communication succeeds, or fails for a specified number of times, a specified callback for completing the transmission is called when calling WM_SetMPDataToPort*. Do not rewrite the memory area that contains the transmission data specified with WM_SetMPDataToPort* until the transmission complete callback is received.

### Priority and Send Queue

Port communication includes four levels (0 to 3) of priority. The data reserved with SetMPDataToPort is processed as a first-in first-out (FIFO) send queue. There are four different send queues according to the priority levels. Unless the high-priority queue becomes empty, data will not be sent out from the lower-priority queue. Communication that is more likely to be processed in real time, such as data sharing, is set to priority 1, while communication that is less likely to be processed in real time is set to priority 3.

With Raw communication, data can be set to the same port number at different priority levels. With Sequential communication, setting the data to various priority levels might cause incompatibility with the order controlled by the sequence number. Specifically, when data of a high-priority level is set later, data will be sent according to the priority levels, but the lower-priority data that was set earlier may not be sent properly. (With the current implementation, it may be sent twice.)

If SetMPDataToPort was called while the send queue is full, WM_ERRCODE_SEND_QUEUE_FULL is returned to the callback, and it will fail. With the current implementation, the total of 64 send packets for all priority levels are kept in queue. However, when the next data is set after the transmission complete callback of SetMPDataToPort, only one level of the send queue is used, and the overflow is less likely to occur with normal usage like this. Data sharing uses up to two levels.

### The Maximum Number of Send Bytes

In MP communications, the parent device determines the maximum number of bytes that can be sent by the respective parents and children at the time when the communication begins. To be specific, prior to WM_StartParent, the maximum number of bytes is set as the argument of the WM_SetParentParameter that configures the parameters of the parent device. The parentMaxSize field of the WMParentParam structure given to WM_SetParentParameter expresses the maximum send size from the parent device to the child device, while the childMaxSize field expresses the maximum send size from the child device to the parent device.

As described in Communication Packet, the maximum send size configured here is the maximum size of the user data that can be sent with the SetMPData–type API. This is absolutely the maximum size when only one packet could be sent with one MP communication. If multiple packets are packed and sent at the same time, corresponding headers and footers must be added to the packet. Therefore, when sending multiple packets at the same time, determine the number of bytes using the following formula:

[The Number of Bytes Used] = [The Total User Data Size to Pack] + [Added Headers and Footers] x ([The Number of Packets to Pack]-1)
[Added Headers and Footers] = 6 bytes (in the case of the parent device) or 4 bytes (in the case of the child device)

There are two limitations on the maximum number of send bytes:

- The number of transmitted bytes on either the parent or child must not exceed 512.

- The amount of time for one MP sequence as calculated from the number of parent and child send bytes must not exceed 5600 microseconds. In other words, the following formula must apply:

96 + (24 + 4 + [The Number of Parent Device Send Bytes] + 6 + 4)*4 + (10 + 96 + (24 + [The Number of Child Device Send Bytes] + 4 + 4)*4 + 6)*[The Number of Child Devices] + 10 + 96 + (24 + 4 + 4)*4 <= 5600
⇔
[The Number of Parent Device Send Bytes] + ([The Number of Child Device Send Bytes] + 60)*[The Number of Child Devices] < 1280

If WMParentParam.KS_Flag is set to TRUE, because internally it adds the number of bytes for Key Sharing, the value that can be set as the maximum number of send bytes is less than that of the above formula. 36 bytes plus 6 bytes (for the header and footer) for the parent device, and 6 bytes plus 4 bytes (for the header and footer) for the child device are allocated internally, so be sure to include those bytes in the calculation.

A script that calculates the communication time limits is available at:

$NITROSDK_ROOT\man\en_US\wm\wm\wm_time_calc.html

### Key Sharing and Data Sharing

For games with lots of real-time communications, you may want to reliably share the same data (positional information, performance information, etc.) at fixed intervals with all the participants. A set of Data Sharing APIs is available for situations like this. Also, a specialized Key Sharing API can be used to share key input. Key Sharing uses Data Sharing internally.

Data Sharing is easy to use. Immediately after MP communications are started by WM_StartMP, initialize Data Sharing by calling WM_StartDataSharing. After that, call WM_StepDataSharing at the top of each game frame to allow data sharing between a parent and a child. If WM_StepDataSharing returns WM_ERRCODE_NO_DATASET, it means one of the communication partners dropped out of the processing, so the game frame update is delayed and there is a wait of one picture frame. If WM_ERRCODE_SUCCESS is returned, it means all members participating in Data Sharing could share the data, so a new game frame is updated with the shared data. By sharing data, synchronous, real-time gaming is easily achieved.

If synchronization fails, when the game frame, for example, is 30 fps in the application, WM_StepDataSharing is called once every two frames. If WM_ERRCODE_NO_DATASET is returned, WM_StepDataSharing must be called at the next frame.

01: WM_StepDataSharing() == WM_ERRCODE_SUCCESS
02: ----
03: WM_StepDataSharing() == WM_ERRCODE_SUCCESS
04: ----
05: WM_StepDataSharing() == WM_ERRCODE_NO_DATASET
06: WM_StepDataSharing() == WM_ERRCODE_SUCCESS
07: ----
08: WM_StepDataSharing() == WM_ERRCODE_SUCCESS
09: ----

The failure should be processed as shown above, and there should not be a frame between the 5th and 6th frames. Otherwise, the correction will not work when there is a one-frame difference between the parent and child.

WM_StartDataSharing should be called immediately after WM_StartMP, and WM_EndDataSharing should be called immediately before WM_EndMP. Delaying the start of data sharing can be achieved simply by not calling WM_StepDataSharing. Alarms or timers are not used in data sharing, and processes are done by calling APIs and send/receive callbacks. Therefore, extra processes or communications are not performed unless WM_StepDataSharing is called even after WM_StartDataSharing. However, there are some restrictions for the timing to call back WM_StepDataSharing for this reason. To have stable data sharing, it is necessary to call WM_StepDataSharing at the earliest possible time after the V-Blank, so that the send data can be set by the time ARM7 starts preparing for the next MP communication (in terms of V-count, 240 for child and 260 for parent).

The send data size that Data Sharing uses is calculated as follows:

[The Parent Device Data Size] = [The Shared Data Size] x [The Number of Devices Sharing the Data(including the parent device)] + 4
[The Child Device Data Size] = [The Shared Data Size]

As a limitation on the API, the parent data size must be 512 bytes or less (it was 256 bytes or less until 2.0 RC1). This means that [the shared data size] * [number of shared devices] must be less than or equal to 508. Also, the shared data size must be an even number. For example, if there are 5 child devices, the shared data size is up to 84 bytes.

(84*6 + 4 = 508 <= 512, 86*6 + 4 = 520 > 512)

When the number of children is 6 or more, the maximum size of shared data is determined by the 5600 s limitation for the required time for communication, as explained in the previous section. For example, if there are 15 child devices, shared data size is up to 12 bytes.

((12*16 + 4) + (12+60)*15 = 1276 < 1280, (14*16 + 4) + (14+60)*15 = 1338 < 1280)

Following is a list of the maximum size of shared data for each number of child devices.

| Number of Child Devices | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maximum shared size by the restriction of the parent’s data size <= 512 bytes. | 254 | 168 | 126 | 100 | 84 | 72 | 62 | 56 | 50 | 46 | 42 | 38 | 36 | 32 | 30 |
| Maximum shared size by the restriction of the required time for communication <= 5600 s | - | - | - | - | - | 70 | 56 | 46 | 38 | 32 | 26 | 22 | 18 | 14 | 12 |

When using this at the same time as a normal SetMPData—type API, multiple packets are packed. Note that when calculating the respective maximum sizes for each parent and child device, only the header and footer portion of the packet (6 bytes for the parent, 4 bytes for the child) must be added.

### Events Triggered by the Wireless Library

Aside from the "operation complete" notification corresponding to the call in one section of the asynchronous API callback, an event notification call from the Wireless Library is also required. The trigger for the callback being called is stored in the value of the WMStateCode enumerated type in the state field of the WM*Callback structure of the callback argument.

Below is a list of WMStateCodes and their descriptions for particular APIs. However, for asynchronous APIs where the WMSetMPData–type API was excluded (because tables of callback functions are kept internally only in API units), try to avoid giving different callback functions in each call to identical APIs.

| WMStateCodes | Enumerated Constant Description |
| --- | --- |
| WMStartParent | WM_STATECODE_PARENT_START An asynchronous notification that the API call is complete. WM_STATECODE_BEACON_SENT The beacon was sent. WM_STATECODE_CONNECTED A child device connected to the parent. At connection time, the following data is sent via callback: - The AID of the child connected to WMStartParentCallback.aid - The MAC address of the child in WMStartParentCallback.macAddress - The user region (the second 24 bytes) of the SSID declared by the child in WMStartParentCallback.ssid WM_STATECODE_DISCONNECTED A child disconnected from the parent. WMStartParentCallback.aid and WMStartParentCallback.macAddress behave as in WM_STATECODE_CONNECTED. |
| WMStartConnect | WM_STATECODE_CONNECT_START An asynchronous notification that the API call is complete. If no more entries are being received because parent‘s entry flag is set to FALSE or the device has reached its maximum number of connections occurs, etc., WM_ERRCODE_NO_ENTRY or WM_ERRCODE_OVER_MAX_ENTRY may be returned in the errcode. WM_STATECODE_BEACON_LOST The beacon of the connected parent device fails to receive 16 times in succession. It is likely that the radio state is bad, but there is no particular need to deal with this. WM_STATECODE_CONNECTED A connection was made with the parent device. At connection time, the following data is sent via callback: - The AID of the child connected to WMStartConnectCallback.aid WM_STATECODE_DISCONNECTED A parent disconnected from a child. WMStartConnectCallback.aid behaves as in WM_STATECODE_CONNECTED. |
| WMStartMP WMStartMPEx | WM_STATECODE_MP_START An asynchronous notification that the API call is complete. WM_STATECODE_MPEND_IND The parent device sends out the MPACK frame and successive MP sequences are finished. Notifies the pointer to the WMMpRecvHeader structure that stores the contents of the frame received from the child in WMStartMPCallback.recvBuf. We recommend using the port receive callback to receive the data. However, since the recvBuf field is defined as a WMMpRecvBuf type, you must forcibly recast the field type. WM_STATECODE_MP_IND The child received the MP frame from the parent. Notifies the pointer to the WMMpRecvBuf structure that stores the contents of the frame received from the parent in WMStartMPCallback.recvBuf. We recommend using the port receive callback to receive the data. If the port was not assigned with the PollBitmap of the MP frame, the errcode is WM_ERRCODE_INVALID_POLLBITMAP. Because this occurs most often when multiple child devices are connected, it should not be handled as an unrecoverable error. Also, if counting the header information and nothing was included in the received MP frame, WM_ERRCODE_NO_DATA is notified as the errcode. Normally, as long as the WM Library is operating, this cannot occur. WM_STATECODE_MPACK_IND The child received the MPACK frame from the parent device. The errcode is WM_ERRCODE_INVALID_POLLBITMAP if not primarily self-designated with the PollBitmap of the MP frame that corresponds to this MPACK. Since this occurs most often when multiple child devices are connected, it should not be handled as an unrecoverable error. Otherwise, if the parent is not notified with the PollBitmap field of the MPACK frame that the Key(Null) response frame was not received, the errcode is WM_ERRCODE_SEND_FAILED. Even if a given time passes after receiving the MP frame, if the MPACK frame could not be received, then this indication occurs, the errcode is WM_ERRCODE_TIMEOUT, and there is a notification. |
| WMSetIndCallback | WM_STATECODE_BEACON_RECV The beacon from the connected parent is received. If WMIndCallback.state was this value, by recasting the type in WMBeaconRecvIndCallback, the GameInfo can be obtained from WMBeaconRecvIndCallback.gameInfoLength, WMBeaconRecvIndCallback.gameInfo, etc. WM_STATECODE_FIFO_ERROR This statecode is sent to the ARM7 when the execution control queue overflows due to a process overload on the ARM7. Treat as a non-recoverable fatal error. WM_STATECODE_DISASSOCIATE Used for debugging. Normally, you can ignore this constant. WM_STATECODE_REASSOCIATE Used for debugging. Normally, you can ignore this constant. WM_STATECODE_AUTHENTICATE Used for debugging. Normally, you can ignore this constant. |
| WMSetPortCallback | WM_STATECODE_PORT_RECV Data is received from the communication partner. The following data is sent via callback: - The AID of the child connected to WMStartConnectCallback.aid - The AID of the send source in WMPortRecvCallback.aid - The pointer to the receive data in WMPortRecvCallback.data - The size of the receive data in WMPortRecvCallback.length - The void* arg given to the argument of the WMSetPortCallback in *.arg WM_STATECODE_CONNECTED Immediately after notification in the callbacks of WMStartParent and WMStartConnect that the connection was established, similar notifications are sent to the receive callbacks of every port. Note that, regardless whether it's a parent or child, WMPortRecvCallback.aid always takes the AID of the connection partner at that time (the child device is fixed at 0, while the parent takes the AID of the connected child). Its own AID is stored in *.myAid (SDK 2.0 PR7 and later). Also, the MAC address and user region SSID (for the parent device) of the respective communication partners are set to *.macAddress and *.ssid. WM_STATECODE_DISCONNECTED Immediately after notification in the callbacks of WMStartParent and WMStartConnect that the connection was terminated, similar notifications are sent to the receive callbacks of every port. The same notes apply to AID as to CONNECTED. Also, the MAC address of the disconnected partner is stored in *.macAddress. |
| WMSetMPData WMSetMPDataToPort WMSetMPDataToPortEx | WM_STATECODE_PORT_SEND Only one kind of STATECODE is notified as the completion callback of an asynchronous API, but because it is an important notification in the communication controls, it is described separately here. In WMPortSendCallback.errcode, the following data is sent via callback: - WM_ERRCODE_SUCCESS when the send succeeded - WM_ERRCODE_SEND_FAILED when the send failed - WM_ERRCODE_SEND_QUEUE_FULL when the send queue was full If the send failed, a bitmap of the AID of the partner that must retry is stored in *.restBitmap. Except for commands that end communication, such as WM_EndMP, WM_End, or WM_Reset, exactly one callback occurs for each API call. Regardless of their success or failure, the memory region of the send data must not be overwritten between the API call and when the callback returns. You can obtain the address of the set send data using WMPortSendCallback.data. If the arg is designated with WMSetMPDataToPortEx, it is passed to *.arg. |

## FAQ

Some of the major questions asked by the WM library users are shown below in question and answer format.

### Initialization process

Q: A valid value is not returned for WM_GetAllowedChannel.

A: WM_GetAllowedChannel does not return a valid value until after WM_Init is called. (that is for NITRO-SDK 2.0 RC1 and later versions. With NITRO-SDK 2.0 FC and earlier versions, it was enabled before calling WM_PowerOn.) If it was called before the initialization, it returns 0x8000, which indicates an error. (NITRO-SDK 2.0 RC2 and later versions)

### Connection process

Q: I don’t know the value to set for WMParentParam.tgid.

A: Ideally it should be a different value every time, even when the power is restored. For an easy and convenient way, a pseudo-random number can be generated by combining the return values of OS_GetVBlankCount and GX_GetVCount. Also, by using the value for seconds or minutes on the RTC, it is possible to guarantee that value to be different for some time, even after the power was restored. For the implementation during which a child reconnects to the parent multiple times, by sending some bits of TGID to the phase information to prevent a child from connecting to the parent with a different phase, a secure connection may be achieved.

Q: When creating a list of parents from the scan result, I find a parent that is sometimes difficult to find.

A: Mix random numbers in the beacon intervals of the parent and the scan intervals of the child. If all parents had the same beacon intervals and if the beacon send timing happened to be immediately after the other parent’s beacon, that parent may be difficult to be found. Also, the processing overhead may be an influence, and the parent’s beacon interval may become multiples of the child’s scan interval. In such cases, a parent may be difficult to be found.

To make improvements, WM_GetDispersionBeaconPeriod and WM_GetDispersionScanPeriod were added in NITRO-SDK 2.0 RC2. Each of them returns random values that are around 200 ms and 30 ms. By setting a value of WM_GetDispersionBeaconPeriod to WMParentParam.beaconPeriod, the frequency of getting the same beacon intervals on the parents can be reduced (set once when starting a parent). Also, reset the return value of WM_GetDispersionScanPeriod to WMScanParam.maxChannelTime every time a child does WM_StartScan to make child scans at different timing intervals. If there is no special reason for changing them, use the values returned by these APIs.

In the future, WM_StartScanEx, which obtains multiple parents at once, will be provided.

Q: The connection with WM_StartConnect is not secure.

A: Check whether WM_Reset was forgotten to be called when retrying the connection, after a failure. If the failure occurred in the middle of the connection sequence, the internal state may have become inappropriate, and WM_Reset must be used before calling WM_StartConnect again, to reset the internal state.

Also, the connection of a child at an unexpected time can be prevented by calling WM_SetEntry at the time of a parent closing entries by children, to disable the entry flag. Check the bit of WM_ATTR_FLAG_ENTRY of gameInfo.gameNameCount_attribute in the beacon before the actual connection is tried by the children, to determine if the parent is accepting entries.

Q: When I end the communication once and use the same parent to reconnect, sometimes it fails.

A: When it scanned and reconnected, there may be some cases where the timing on the child’s reconnection process was too early. The child may pick up the old beacon from the parent before the ending and try to reconnect to the parent. Check the beacon information before the connection, to see if the new connection has started. There are different methods to find the status of the parent from the beacon information, such as including the parent’s phase information to userGameInfo, or checking the change of TGID. When rescanning and reconnecting after multiboot, check WM_ATTR_FLAG_MB of gameInfo.gameNameCount_attribute to determine if it is the multiboot parent.

If not doing rescanning, update TGID by the predetermined rule when reconnecting. Use some of the bits in TGID for the phase information of the parent, and rewrite that section of WMParentParam and WMBssDesc on parent and child, to be sure. When doing so, note that the child cannot be reconnected if by mistake the parent changed the channel with the connection immediately before.

Q: It is not a multiboot parent, but when starting up the DS wireless play parent, mb_child_simple.srl (with NITRO-SDK 2.0 RC2, mb_child.srl with NITRO-SDK 2.0 RC1 and earlier versions) reacts. (“GameInfo Receiving…” keeps appearing.)

A: Check if the multiBootFlag field of the WMParentParam structure specified by WM_SetParentParam is set to a value other than 0. Do not enable multiBootFlag if it is not the parent that sends out the beacon for multiboot, to wait for a multiboot child.

### General MP communications

Q: What should I do to send out the data with the shortest delay for MP communication?

A: With the frame synchronization communication mode, the MP communication starting process for the beginning of the frame is performed when V-Count is at 260 lines. When a child receives the MP frame from the parent, the transmission data should already be set, so the transmission data setting process starts a little earlier at 240 lines. Therefore, if you want to reduce the latency as much as possible, call WM_SetMPDataToPort at 260 lines with the parent, and at 240 lines with a child. However, it is not guaranteed to be sent out immediately because there are some unpredictable delays between the time the API is called from ARM9 and the time the wireless driver of ARM7 processes. And besides that, if there was already some other data in send queue, that data is sent out first.

For the second or later MP communication, in continuous communication mode or frame synchronization mode, the next MP communication starts with the parent, and the next transmission data is set with a child when MP communication immediately before it has completed. In that case, the setting for the next MP communication is done in time by starting WM_SetMPDataToPort in the transmission completion callback and port receiving callback that are started as a result of the MP communication immediately before that. Note that it may not be in time depending on the status of the ARM7 wireless driver.

In the future, the API will be provided to change the default value of 260 lines and 240 lines.

### Data Sharing

Q: WM_StepDataSharing frequently returns WM_ERRCODE_NO_DATASET.

A: There are a few possibilities. If the parent or the child always succeeds, there is a possibility that the step is failing because the process with the one that keeps succeeding failed, and that the other is waiting for the process failure. If WM_StepDataSharing is set to be called at every frame, and it always fails once every two frames, check if doubleMode of WM_StartDataSharing is set to TRUE. If WM_StepDataSharing is set to be called once every two frames and it fails on a regular basis, there may be a problem with the retry process in case WM_ERRCODE_NO_DATASET was returned. Check to see if the next WM_StepDataSharing is called in the frame immediately after the failure.

If it fails with parent and child randomly at about the same frequency, there is a possibility that WM_StepDataSharing is called at bad timing. Be sure to call WM_StepDataSharing at the earliest possible time after V-Blank. WM_StepDataSharing calls WM_SetMPDataToPort internally, but in order to perform data sharing with the least MP communication frequencies, it requires data to be on every MP communication. Therefore, as explained in the previous item, data sharing may not be stable because of the communication timing, if it is not 260 lines with the parent and 240 lines with a child. This is the same with Key Sharing because it performs data sharing internally.

Q: The code does not work properly when pausing with WM_EndDataSharing and restarting with WM_StartDataSharing.

A: WM_EndDataSharing is designed to be called as a series of processes for ending communication, and it may cause a problem if the termination and restarting were performed in a row. If you want to pause the data sharing, set a flag in the shared data to set the timing to pause with both parent and child, and stop calling WM_StepDataSharing. Unless WM_StepDataSharing is called, the extra processing time or communication regarding data sharing will not be generated. Note that the last data set before pausing is delivered after restarted.

In the future, the API for pausing will be provided.

Q: Can the shared data size be changed by using the same port?

A: This is planned to be implemented in the future, but it cannot be done at this point. Call WM_StartDataSharing and WM_EndDataSharing once for starting communication and ending communication respectively, and use one port for data sharing of the same setting during the communication.

Instead, the same process is achieved by performing two sets of data sharing with different shared sizes at different ports, and switching them. Precautions for switching are as shown above.

### Others

Q: The communication stops sometimes for unknown reasons.

A: There may be various causes, such as the destruction of memory in the application. Check for the following:

- Are you using a process that takes a long period in the callback? Callback is in the interrupt handler, so it may be in interrupt prohibited state and the wireless driver of ARM7 may be waiting for the callback to complete. It would cause negative effects in some areas, so avoid using processes that take several milliseconds.

- Are there multiple levels of nesting of function callbacks within a callback? Or, calling a function such as OS_Printf that consumes many stacks in a deeper level of nesting? Be sure to reduce the consumption of stacks, because IRQ stack used while the callback is executed is not very big. If it freezes during debug output, the situation may be improved by using OS_TPrintf instead of OS_Printf.

- Is WM_StartConnect of a child called apart from WM_StartMP and WM_StartDataSharing? These functions on a child should be called in a series, due to the current restrictions on their implementation.

Q: Is there anything that requires special attention when debugging the wireless communication portion?

A: First, be sure to allocate enough time for debugging wireless. It may seem to be working properly, but a problem that occurs once every a few dozen times is very common.

For debugging, change to fixed channels by temporarily disabling the automatic channel selection feature, and start multiple groups of parent and children on the same channel. By repeating this test, you may have a better chance of recreating the problem.

## Important Notes for Recent Releases

Several important changes to the wireless API are explained below. Note that some changes are not obvious when compiling.

### Changes to Receive Processing (SDK 2.0 PR4 and Higher)

Before PR4, in the operation samples of the wireless API, data was received via indication functions. For children, the indication function was for the WM_STATECODE_MP_IND value that comes to the WM_StartMP callback. For parents, the indication function was for the WM_STATECODE_MPEND_IND value that comes to the WM_StartMP callback. The content of the receiving frame payload was passed to the indication function, enabling the receive data operation to be executed.

Along with allowing multiple packets in one payload, however, came instances where compatibility was not maintained. Though conventional processing easily handles communications with only one packet, it cannot handle multiple packets well. We recommend that the application's receive processing routine be modified to receive communications in packet units using WM_SetPortCallback. Beginning with SDK 2.0 PR6, $NITROSDK_ROOT\build\demos\wm\simple-1 uses the port receiving callback.

### Changes to the Key Sharing Packaging (SDK 2.0 PR4 and Higher)

In the past, Key Sharing was processed in a completely different layer from normal communications. With the packaging of Data Sharing, however, the packaging for Key Sharing changed to use Data Sharing.

The API interface maintains full backward compatibility, but limitations to the timing for calling WM_GetKeySet were added. In frame sync communication mode, the call must be made sufficiently earlier than the frame's MP communication due to the internal call of WM_StepDataSharing by WM_GetKeySet. Make the call as soon after V-Blank as possible, such as near the beginning of the V-Blank interrupt processing. In the current packaging, the set processing for the child's Key response data starts when the V count is 240.

### Changes to the Notification Error Code for Transmission Failure (SDK 2.0 PR6 and Higher)

In the WM_STATECODE_MPACK_IND indication returned in the WM_StartMP callback and in the WM_SetMPData API callback, WM_ERRCODE_FAILED were used for transmission failure notification, but this was changed to WM_ERRCODE_SEND_FAILED.

### New Notification: WM_ERRCODE_INVALID_POLLBITMAP (SDK 2.0 PR6 and Higher)

For children, the notification WM_ERRCODE_INVALID_POLLBITMAP is now used for indications from WM_STATECODE_MP_IND to _MPACK_IND returned for WM_StartMP callbacks. This enables a child to ignore MP frames not sent to it. This happens frequently when more than one child is connected, so it may not be handled as a recoverable error.

### Notification Changes for Port Receiving Callback Connections and Disconnections (2.0 PR7 & Higher)

For a child's port receiving the callback WMPortRecvCallback.aid, 0 is always entered in WM_STATECODE_PORT_RECV to represent the parent AID. In the past, the child's own AID was entered for the WM_STATECODE_CONNECTED notification, and an indefinite value was entered for WM_STATECODE_DISCONNECTED, but this was changed so 0 now represents the parent's AID. The child's own AID is held in the new WMPortRecvCallback.myAid.

### Changes to the WMParentParam and WMGameInfo Structures (from 2.0 RC1)

The WMParentParam structure, which holds parent info, was changed. To eliminate confusion about the byte order, the ggid field was changed from a u16 array of length 2 to a u32 scalar variable. As a result of this change, a dummy field was inserted in front of the ggid to meet the requirements for 4-byte alignment. For the same reason, the ggid field in WMGameInfo was changed to a u32.

In applications based on the wc.c demo, you should change the section that configures parent information and the section that confirms the scanned ggids so that they compare a single u32 variable instead of an array of u16.

### Change in MAC address type (from NitroSDK Version 2.0 RC1)

To eliminate confusion about the byte order, MAC addresses are no longer treated as a u16 array of length 3. Instead they are treated as a u8 array of length 6. All API callback processes that handle MAC addresses must be rewritten. Note that fields named bssid were changed in addition to those named macAddress.

### Corrected the wireless demo (NITRO-SDK 2.0 RC2 and as needed)

The wireless demo included in the SDK demos is updated whenever a problem is found. If you are using wc.c or wmhigh.c, included in the wireless demo, to be embedded in the application as a base, check the version of SDK you have used as a base to see if there are any changes with these files. Note that some problems, such as the termination process, were found in wmhigh.c, and that the file was updated.

Also, demos/wm/dataShare-Model/src/wmhigh.c and demos/mb/multiboot-Model/common/src/wmhigh.c were merged and became demos/wireless_shared/wh/wh.c. Similarly, simple-1/src/wc.c and dataShare-1/src/wc.c under demos/wm were merged, and became demos/wireless_shared/wc/wc.c.

© 2004 Nintendo

The contents of this document cannot be duplicated, copied, reprinted, transferred, distributed or loaned in whole or in part without the prior approval of Nintendo.